Design and Performance

User Manual

Group 31

September 18, 2017

Contents

Included in the Package	3

Description of Parts	4

How to Use	4

Enter into Infrared Remote Control Mode	5

Enter into Bluetooth Mode	5

Enter into Ultrasonic Obstacle Avoidance Mode	6

Additional Function	6

Troubleshooting	7

Specification	7

Cautions	7

Included in the Package

- A ship-shaped drawing robot.

- Two 3.7V rechargeable batteries.

- A battery charger.

- An infrared remote controller.

Description of Parts

| Parts | Function |
| --- | --- |
| Green LED light | Show operation state via lighting and twinkling |
| Ultrasonic detector | Measure distance |
| Step motor | Spin two wheels |
| Wheel | Control the robot movement |
| Buzzer | Play music and warn |
| Bluetooth module | Transmit and receive the Bluetooth signal |
| Infrared receiver | Receiver the infrared signal |
| Micro servo | Control the pen |
| Pen | Draw the diagram you want |

Table 1

How to Use

Enter into Infrared Remote Control Mode

- Install two batteries.

- Press the corresponding key in infrared remote controller towards infrared receiver according to keyboard functions table.

- If two wheels are rotating or the robot is playing music, the green LED light will keep lighting.

- If two wheels stop, the green LED light will twinkle once.

- If the robot enters into Bluetooth mode, the green LED light will twinkle twice.

- If the robot enters into ultrasonic obstacle avoidance mode, the green LED light will twinkle three times.

- If infrared receiver receives the wrong signal, the buzzer will warn.

- Keyboard Functions Table

|  | Key | Function |
| --- | --- | --- |
| Draw a diagram | 1 | Draw a triangle |
|  | 2 | Draw a circle |
|  | 3 | Draw a square |
|  | 4 | Draw a rectangle |
| Play music | 5 | Play a song |
| User custom drawing | ▲ | Walk forward |
|  | ▼ | Walk backward |
|  | ◀ | Turn left |
|  | ▶ | Turn right |
|  | OK | Stop |
|  | 7 | Lift the pen |
|  | 8 | Put down the pen |
| Switch mode | * | Enter into Bluetooth mode |
|  | # | Enter into ultrasonic obstacle avoidance mode |

Table 2

Enter into Bluetooth Mode

- Ensure that you are already in infrared remote control mode.

- Press * key to switch to Bluetooth mode.

- If you can see the green LED light twinkle twice, you are in Bluetooth mode.

- Download a Bluetooth software on your device.

- Open Bluetooth on your device and search the robot Bluetooth named HC06, the default password is 0000.

- After successfully connecting to Bluetooth, enter the appropriate letter in the dialog window or press appropriate key according to the Bluetooth functions table.

- You will receive a response after every instruction.

- Bluetooth Functions Table

|  | Key | Function |
| --- | --- | --- |
| Draw a diagram | T | Draw a triangle |
|  | C | Draw a circle |
|  | Z | Draw a square |
|  | J | Draw a rectangle |
| Play music | M | Play a song |
| User custom drawing | W | Walk forward |
|  | B | Walk backward |
|  | L | Turn left |
|  | R | Turn right |
|  | S | Stop |
|  | Q | Lift the pen |
|  | D | Put down the pen |
| Switch mode | U | Enter into ultrasonic obstacle avoidance mode |
| Exit | E | Exit Bluetooth mode and back to infrared remote control mode |

Table 3

Enter into Ultrasonic Obstacle Avoidance Mode

- You can enter into this mode via infrared remote control mode or Bluetooth mode.

- Robot in this mode can automatically avoid obstacles to walk.

- This mode doesn’t support music.

Additional Function

- User custom drawing.

- Support Bluetooth mode.

- Realize ultrasonic obstacle avoidance.

- Play music.

Troubleshooting

| Issue | Possible Problem | Solution |
| --- | --- | --- |
| Wheel doesn’t rotate. | Something wrong with the connection. | Check that the stepper motor and the wheel are connected correctly |
| Infrared remote control doesn’t work. | The power of controller is running out. | Charge the battery fully. |
|  | Something wrong with the receiver connection. | Check the receiver connection. |
| Green LED light doesn’t work. | Something wrong with the connection or the light is burned out. | Check the connection. If the connection is correct, change a light. |

Table 4

Specification

| Item | Description |
| --- | --- |
| Operation Temperature | -20℃~50℃ |
| Dimension | 37.5cm×13cm×26.5cm |
| Operating Voltage | 7.4V |

Table 5

Cautions

- DO NOT repair or disassemble it by yourself.

- DO NOT be near water and fire.

- DO NOT drop it.

- DO NOT put it in high voltage circumstance.

- It can only be used within the permissible temperature range.

- Must remove the batteries when it is not in use.